# Purpose

This procedure documents Wannon Water’s standard requirements for response to an Alarm triggered by the Vehicle Safety System (VSS), also referred to as the Smartrak System.

# Scope

This procedure applies to:

- Vehicle Safety System Manager.

- Vehicle Safety System Alarm Security Monitoring Centre (SMC) provider.

- All employees and/or contractors using a Wannon Water vehicle with VSS installed.

# Procedure

The VSS will generate alarms when these specific events occur.

- Vehicle Impact

- Vehicle Rollover

- Pendant Duress Button Activation

- Dash Duress Button Activation

The alarms will be configured to automatically send alerts via several methods:

- Email alert to pre-configured addresses.

- SMS alert to pre-configured mobile phone numbers.

- The Smartrak application will alert all logged-in users via an audio alert and visual notification.

- Integration to alarm SMC software.

## Alarm Monitoring and Response

Alarms will be transmitted electronically to the SMC.  The SMC response will be in accordance with of this Procedure.

## Wannon Water Alarm Monitoring

Alarms will also be transmitted electronically to the Vehicle Safety System Manager.  The Vehicle Safety System Manager response will be in accordance with this Procedure.

## SMC Alarm Signal Types and Notifications

Type 1 Vehicle Impact Alarms

This alarm is activated automatically.  No user activation is required.

Type 2 Vehicle Roll-over Alarms

This alarm is activated automatically.  No user activation is required.

Type 3 Pendant Duress Button Activation Alarms

This alarm is activated by holding both pendant buttons simultaneously for at least one second. Alarm signals are indicated by the blinking transmission LED.

Type 4 Dash Duress Button Activation Alarms

This alarm is activated by holding the dash duress buttons down for at least one second. Alarm signals are indicated by the blinking transmission LED.

## Photos of alarm activation devices

| Dash Mounted Alarm Button | Pendant Alarm Device |
| --- | --- |

## Alarm Response Process

Alarm signals are transmitted electronically to the SMC, with a copy also sent to the Vehicle Safety System Manager the Fleet & Facilities Coordinator.

### Procedure for SMC (Response)

The following outlines steps that occur for an alarm event(see Figure 1):

- Upon activation of the alarm, SMC will call the contacts displayed on the vehicle resource screen in order as follows:

  - Driver (where i-Button is attached); or

  - Custodian, and if no answer within business hours;

  - BH Contact

- Where the driver or custodian answers, ascertain the nature of the emergency.

- If an emergency is confirmed, call triple-zero (000) immediately, and the appropriate emergency service asked to respond to the drivers last known location.

- If SMC cannot make contact with any contacts detailed above, call triple-zero (000) immediately, and police asked to respond to the drivers last known location.

- SMC is to call the Wannon Water Vehicle Safety System Manager to explain actions taken and the alarm status at the conclusion of steps 1 – 4 inclusive.

Figure 1: Flowchart of SMC Response

### Procedure for Vehicle Safety System Manager (Monitoring and Records)

The following outlines steps that occur for an alarm event as (see Figure 2):

- The system will generate an automatic notification of an alarm occurrence via SMS and/or email.

- After SMC have followed their procedure, they will contact the Vehicle Safety System Manager to advise the actions taken.

- Take appropriate action, which may include calling the manager of the driver or custodian.

- Ensure the event alarm is closed in the VSS system.

Figure 2: Flowchart of Wannon Water Monitoring and Records

## Information for drivers

- Upon activation of the alarm, SMC will attempt to make contact with the driver and, where the Driver has not fobbed on or is unable to answer the call, the Custodian, and during business hours the BH Contact.

- Where you take a call from SMC and you do not require emergency services, inform the SMC operator.

- If emergency services are required, request SMC to send the appropriate emergency service.

- Note: If SMC cannot make contact with you, triple-zero (000) will be called, and police asked to respond to your last known location.

- Note: In all alarm events, the Vehicle Safety System Manager will be notified of the event and take appropriate action.

## Alarm Incident Records

- For all alarms where an emergency service response is required, the vehicle driver or their designated line manager must complete and submit an incident, as soon as practical after the incident, for action in accordance with the Wannon Water Incident Response & Reporting Procedure.

In addition, an “Emergency Log Report” will be recorded in Records Manager and provide to the Vehicle Safety System Manager each month for review.

## Notification of Alarm Incidents Internally

- Refer to Wannon Water’s Emergency Management Plan incident trigger and communications protocols and Incident Response & Reporting Procedure to determine the need for internal emergency response escalations in the case of a duress alarm incident being classified as an emergency, or if an incident results in an injury to an employee or other person.

# Roles and responsibilities

| Position | Roles and responsibilities |
| --- | --- |
| Vehicle Safety System Manager | This role is fulfilled by the Manager Corporate Contracts and Projects and is responsible for the overarching management of the VSS and its monitoring and reporting processes. |
| Coordinator Fleet and Facilities | Provide backup for the Vehicle Safety System Manager |
| Managers | Branch Managers and above with a private use entitlement under their employment contract are responsible for ensuring that any private users of their vehicle are familiar with the Vehicle Safety System Procedures. |
| All Employees | Responsible for ensuring that they are familiar with the Vehicle Safety System Procedures prior to operating a Wannon Water vehicle. |

# Definitions

| Term | Means |
| --- | --- |
| Vehicle Impact Alarm | Means the Vehicle Safety System has detected a rapid deceleration of the vehicle in space and time during a given journey. |
| Vehicle Rollover Alarm | Means the Vehicle Safety System has detected the orientation of the vehicle in relation to horizontal plane has changed beyond a pre-configured angle during a given journey. |
| Pendant Duress Alarm | Means the portable duress alarm device has been triggered manually by a user for assistance. |
| Dash Duress Alarm | Means the vehicle’s dash-mounted duress alarm device has been triggered manually by a user for assistance. |
| Vehicle Safety System | Vehicle Safety System; also referred to as the “Smartrak System”, means all the hardware and software used by Wannon Water to track and communicate vehicle locations, inclusive of safety duress alarms functions and associated internal or external monitoring centres. |
| Duress Alarm Information | The Alarm message information will contain at a minimum: - The name of the registered driver recorded in the booking system at that time. - The registered mobile phone number of the registered driver recorded in the booking system at that time. - The most recent location, as GPS coordinates, recorded for the vehicle at the time an alarm is logged. |
| Security Monitoring Centre (SMC) | The SMC or Security Monitoring Centre is Wannon Water’s authorised monitoring agent for coordinating a first response in relation to all duress alarms generated by the Vehicle Safety System. Note: The SMC function may be varied from time to time by approval of the Safety System Manager so as to be provided by either an external, third-party contractor, or by internal Wannon Water resources, subject to regular performance reviews of this monitoring function. |
| Vehicle Safety System Manager | The Manager Corporate Contracts and Projects or their delegate. |
| Driver | The driver of the vehicle as indicated by the iButton “Fob” system |
| Custodian | The custodian of the vehicle as recorded in the Smartrak resource information |
| BH Contact | The contact recorded in the Smartrak resource information for business hours contact |

# Governance

| Parent policy/standard | Zero Harm Policy |
| --- | --- |
| Associated procedures/standards | - Vehicle Safety System Manual - Emergency Management Plan - Incident Reporting and Response Procedure - Motor Vehicle Management Procedure |
| Relevant legislation | - AS 2201.2-2004 Australian Standard for Intruder Alarm Systems, Part 2: Monitoring Centres - Working Alone, WorkSafe Victoria Information Sheet, May 2011. http://www.worksafe.vic.gov.au/__data/assets/pdf_file/0006/9555/WS_MIA_WORKING_ALONE_WEB.pdf |
| Approval | Branch Manager Corporate Services |
| Procedure owner | Fleet and Facilities Coordinator |
| Content enquiries | Fleet and Facilities Coordinator |

# Document version history

| Version | Changes made to document |
| --- | --- |
| 4 | - Updated to the new so:control template - Minor administrative changes |